rPPG Signal Estimation for Drunkenness Detection Using RGB Videos: A Literature Review

- INTRODUCTION

- METHODOLOGY FOR PAPER SELECTION

The literature search was conducted across multiple academic databases including IEEE Xplore, ACM Digital Library, arXiv, and Google Scholar. Primary keywords included: "remote photoplethysmography", "rPPG", "drunk detection", "intoxication detection", "driver monitoring", "deep learning", "transformer", "physiological measurement", and "facial video analysis". The search focused on publications from 2018-2024 to capture recent deep learning advances.

Selection criteria for papers: (1) address rPPG signal extraction or intoxication detection, (2) employ deep learning or advanced AI techniques, (3) provide experimental validation with quantitative results, (4) discuss datasets and evaluation metrics.10 papers were selected representing diverse aspects: foundational rPPG techniques [5][7], state-of-the-art architectures [2][9], benchmark toolboxes [6], driving scenario applications [3][4], and intoxication detection systems [1][8][10].

- KEY FINDINGS

A. rPPG Fundamentals and Traditional Methods

Remote photoplethysmography detects blood volume pulse through subtle skin color changes using standard cameras [7]. Traditional approaches employ Independent Component Analysis (ICA) and chrominance-based methods (CHROM, POS) for signal separation [5][7]. However, these hand-crafted methods struggle with motion artifacts and lighting variations common in real-world scenarios.

B. Deep Learning Architectures

Modern rPPG research has moved towards deep learning from start to end. Convolutional approaches include DeepPhys (two-branch 2D CNN with attention), PhysNet (3D CNN), and TS-CAN (temporal shift operations for efficiency) [6]. Transformer-based architectures like PhysFormer capture long-range temporal dependencies, demonstrating superior cross-domain performance [6]. Spiking-PhysFormer reduces energy consumption by 10.1% while achieving 2.80 bpm MAE (using neuromorphic), critical for battery-constrained devices [9].

PhysVLM addresses domain generalization through vision-language models (VLM) that extract physiological features such as heart rate, and help reduce its Mean Absolute Error (MAE) from 5.85 to 2.85 bpm on challenging datasets [2]. This approach focuses on periodic features rather than visual ones, like skin tone.